ECOPROSUS TECHNICAL DOCUMENTATION

ESP32 Node 01 — FC-28 Soil Moisture Monitoring System

FC-28 Soil Moisture Sensor • 0.96″ I²C OLED • Wi-Fi • True UTC NTP • MQTT JSON • Two-Way Handshake ACK

1. Project Overview

This technical document details the implementation of ECOPROSUS ESP32 Node 01, designed for precision soil moisture monitoring and IoT telemetry. The system reads analog and digital signals from an FC-28 soil moisture sensor probe, presents live telemetry on a 0.96″ SSD1306 OLED display, and transmits structured JSON telemetry to a centralized Raspberry Pi MQTT broker every 30 seconds. A closed-loop ACK (acknowledgment) mechanism provides positive hardware indication via dedicated LED patterns.

| System Parameter | Configuration & Details (Supplied Sketch) |
| --- | --- |
| Target Device / Architecture | ESP32 DevKit (3.3V Logic) |
| Device Identifier (JSON) | ESP32_Node_01 |
| Primary Sensor | FC-28 Soil Moisture Sensor (Analog AO + Digital DO) |
| Sensor Calibration Baseline | Dry = 4095 ADC (0% Moisture) ¦ Wet = 1967 ADC (100% Moisture) |
| Display Interface | SSD1306 128×64 I²C OLED (Address: 0x3C; SDA: GPIO 21, SCL: GPIO 22) |
| Visual Indicators | TX LED: GPIO 18 (Double-Blink on TX) ¦ ACK LED: GPIO 23 (3× Blink on ACK) |
| Wi-Fi Network / SSID | SSID: Ecopro ¦ Non-blocking reconnect loop (5000 ms retry interval) |
| MQTT Broker Endpoint | 172.16.0.132:1883 (Raspberry Pi Gateway) |
| Sensor Telemetry Topic | ecoprosus/esp32-01/soil/moisture |
| ACK Subscription Topic | ecoprosus/esp32-01/ack (Awaits 'ACK' payload from Pi / Node-RED) |
| LWT / Status Topic | ecoprosus/esp32-01/status (Retained LWT: 'offline'; Connected: 'online') |
| Telemetry Cadence | Every 30 seconds (30,000 ms interval) |
| Local Time Sync & Display | True UTC system clock via pool.ntp.org; manual +5:30 IST shift for display |

2. System Architecture & Communication Handshake

The node employs a non-blocking asynchronous architecture allowing concurrent sensor sampling, display animation, MQTT socket maintenance, and two-way handshake validation:

• Upstream Telemetry Path: FC-28 Sensor → ESP32 ADC/GPIO → JSON Serialization → Wi-Fi STA → MQTT Broker (Raspberry Pi 172.16.0.132:1883) → Node-RED Flow.

• Closed-Loop Handshake Path: Node-RED verifies incoming packet → Publishes 'ACK' to ecoprosus/esp32-01/ack → ESP32 callback triggers blinkACKLED() (GPIO 23 blinks 3 times).

• Real-Time OLED Display: Renders scrolling corporate title banner, real-time Wi-Fi/MQTT connection states, moisture percentage, soil classification ('dry'/'wet'), and raw ADC integer.

• True UTC Time Architecture: ESP32 internal clock synchronizes to pool.ntp.org with zero GMT offset, ensuring strict ISO-8601 UTC timestamps (YYYY-MM-DDTHH:MM:SSZ) in JSON while dynamically applying IST (+5:30) for local serial monitoring.

3. Complete Hardware Wiring & Pin Mapping

The table below provides exact pin-to-pin connections for ESP32 Node 01. Ensure common ground between all modules and use series current-limiting resistors for LEDs.

| Module / Component | Module Terminal | ESP32 GPIO / Pin | Wiring Specification & Functional Role |
| --- | --- | --- | --- |
| FC-28 Soil Sensor | AO (Analog Out) | GPIO 34 | Analog input (ADC1_CH6). Reads 12-bit voltage (0–4095). |
| FC-28 Soil Sensor | DO (Digital Out) | GPIO 25 | Digital threshold input. Logic HIGH = Dry, Logic LOW = Wet. |
| FC-28 Soil Sensor | VCC | 3V3 | Powered from 3.3V rail to prevent exceeding ADC max voltage. |
| FC-28 Soil Sensor | GND | GND | Common system ground. |
| 0.96″ SSD1306 OLED | SDA | GPIO 21 | I²C Serial Data (shared bus, hardware Wire). |
| 0.96″ SSD1306 OLED | SCL | GPIO 22 | I²C Serial Clock (Wire.begin(21, 22)). |
| 0.96″ SSD1306 OLED | VCC / GND | 3V3 / GND | 3.3V logic power and common ground. |
| TX Indicator LED | Anode (+) | GPIO 18 | Connected via 220–330 Ω series resistor. Double-blinks on TX. |
| TX Indicator LED | Cathode (−) | GND | Common system ground. |
| ACK Indicator LED | Anode (+) | GPIO 23 | Connected via 220–330 Ω series resistor. Blinks 3× upon ACK reception. |
| ACK Indicator LED | Cathode (−) | GND | Common system ground. |

CRITICAL HARDWARE WIRING GUIDELINE: • GPIO 34 is an Input-Only pin on ESP32 (ADC1) without internal pull-up/down resistors, ideal for analog sensors. • Do NOT power the FC-28 sensor from 5V (VIN); applying 5V analog output into GPIO 34 can damage the ESP32 ADC pin. • Series resistors (220 Ω to 330 Ω) are strictly required for GPIO 18 and GPIO 23 to avoid over-current damage.

4. Visual LED Signaling & Sequence Logic

The node features explicit visual feedback confirming outgoing telemetry transmission and confirmed broker delivery:

| Indicator | Pin | Physical Sequence | Trigger Condition & Behavioral Notes |
| --- | --- | --- | --- |
| TX LED | GPIO 18 | Double-Blink: ON 300ms → OFF 200ms → ON 300ms → OFF | Triggers immediately at the start of each 30s publish cycle before the JSON payload is dispatched. |
| ACK LED | GPIO 23 | Triple-Blink: 3 × (ON 200ms → OFF 200ms) | Triggers when MQTT callback receives an incoming payload on 'ecoprosus/esp32-01/ack' matching string 'ACK'. |

5. MQTT Topics & JSON Telemetry Specification

All data exchanged over the MQTT broker adheres to strict JSON document standards using Benoit Blanchon's ArduinoJson library (v6/v7). The MQTT client buffer is explicitly allocated to 512 bytes to prevent frame truncation.

MQTT Topics Architecture

| Direction | MQTT Topic | QoS / Flags | Description |
| --- | --- | --- | --- |
| ESP32 → Broker | ecoprosus/esp32-01/soil/moisture | QoS 0 | Publishes 30s sensor telemetry JSON payload. |
| ESP32 → Broker | ecoprosus/esp32-01/status | QoS 0, Retained | Device lifecycle tracking (Publishes 'online' on connect, LWT sends 'offline'). |
| Broker → ESP32 | ecoprosus/esp32-01/ack | QoS 0 | ESP32 subscribes; awaits string payload 'ACK' from Node-RED. |

JSON Payload Schema (ecoprosus/esp32-01/soil/moisture)

Example JSON structure published every 30 seconds:

{ "device_id": "ESP32_Node_01", "timestamp": "2026-10-22T07:05:30Z", "readings": { "soil_moisture_pct": 68.4, "soil_moisture_raw": 2420 }, "status": { "sensor": "wet", "reading_valid": true } }

6. FC-28 Calibration & Mathematical Model

The FC-28 sensor's analog output voltage decreases inversely with soil moisture. The onboard ADC generates 12-bit integer readings (0 to 4095). In the firmware, soil moisture percentage is computed with inverse linear interpolation:

Moisture Percentage (%) = [(DRY_VALUE - Analog_Raw) / (DRY_VALUE - WET_VALUE)] × 100.0

| Parameter | Calibrated Value | Physical Condition & Behavior |
| --- | --- | --- |
| DRY_VALUE | 4095 | Completely dry probe in free air (maximum resistance, ADC saturation). 0.0% moisture. |
| WET_VALUE | 1967 | Probe fully immersed in water / saturated soil substrate. 100.0% moisture. |
| constrain() function | 0.0% to 100.0% | Clamps readings to prevent out-of-bounds percentages caused by sensor variance. |
| Digital DO Pin (GPIO 25) | HIGH = dry / LOW = wet | Hardware comparator set via onboard potentiometer for discrete threshold trigger. |

7. Software Dependencies & Libraries

The project requires the following standard Arduino/ESP32 core libraries:

| Library | Author / Provider | Purpose in Project |
| --- | --- | --- |
| WiFi.h | Espressif ESP32 Core | Non-blocking Wi-Fi station management. |
| PubSubClient.h | Nick O'Leary | MQTT client for broker connection, LWT, and subscription. |
| ArduinoJson.h | Benoit Blanchon | JSON document construction, serialization, and memory management. |
| time.h | ESP32 Core / standard C | NTP synchronization and true UTC ISO timestamp formatting. |
| Wire.h | ESP32 Core | Hardware I²C master driver on GPIO 21 (SDA) and GPIO 22 (SCL). |
| Adafruit_GFX.h | Adafruit | Core graphics library for text rendering and coordinate positioning. |
| Adafruit_SSD1306.h | Adafruit | Hardware driver for 128×64 monochrome OLED. |

8. Commissioning Procedure & Field Verification Checklist

☐  Verify FC-28 VCC is connected to 3.3V (NOT 5V) and AO is wired to GPIO 34.

☐  Verify TX LED (GPIO 18) and ACK LED (GPIO 23) each have 220–330 Ω series resistors to ground.

☐  Verify I²C lines (SDA to GPIO 21, SCL to GPIO 22) and check that OLED address matches 0x3C.

☐  Verify Wi-Fi network 'Ecopro' is active and reachable within ESP32 range.

☐  Confirm Mosquitto MQTT broker on Raspberry Pi (172.16.0.132:1883) is active and accepting connections.

☐  Ensure Node-RED flow subscribes to 'ecoprosus/esp32-01/soil/moisture' and publishes 'ACK' to 'ecoprosus/esp32-01/ack'.

☐  Open Serial Monitor at 115200 baud to observe NTP sync, IP assignment, and periodic 30-second cycles.

☐  Observe TX LED double-blink during publish and ACK LED triple-blink upon Node-RED response.

☐  Confirm OLED screen scrolls 'Ecoprosus' and displays live Wi-Fi, MQTT, Moisture %, and ADC values.

9. Troubleshooting Guide

| Observed Symptom | Root Cause Analysis | Resolution Procedure |
| --- | --- | --- |
| OLED Remains Blank / Fails to Init | I²C address mismatch or missing pull-ups. | Run I²C scanner. Confirm address is 0x3C. Check SDA (GPIO 21) and SCL (GPIO 22). |
| Moisture Reads 0.0% / ADC 4095 Constantly | Disconnected probe or probe in air. | Check jumper connection to AO pin (GPIO 34). Test by submerging probe in water. |
| Wi-Fi Displays 'OFFLINE' | SSID or password mismatch, 5GHz network. | ESP32 only supports 2.4 GHz Wi-Fi. Verify credentials for 'Ecopro'. |
| MQTT Displays 'BROKER OFF' | Broker service down or subnet isolation. | Ping 172.16.0.132 from network. Check 'systemctl status mosquitto' on Raspberry Pi. |
| No ACK Triple-Blink on LED | Node-RED not responding within 2.0s or topic mismatch. | Verify Node-RED publishes exact string 'ACK' to 'ecoprosus/esp32-01/ack'. Check GPIO 23 resistor/LED polarity. |
| NTP Displays 'NTP Sync...' | Gateway blocking UDP port 123. | Verify router has active internet connection allowing UDP NTP requests to pool.ntp.org. |